| SN | Name of the Experiment |
| --- | --- |
| 1 | Write a program to find the GCD and LCM of a given two bytes in a memory locations |
| 2 | Write a program to convert the given BCD number into its equivalent seven segment value. |
| 3 | Toggling of p1.5 with the delay of 5ms without interrupts using timer 1 |
| 4 | Toggling of p1.2 with the delay of 5ms with interrupts using timer 0 |
| 5 | Count no. 0f pulses without interrupts using counter 0 |
| 6 | Count no. 0f pulses without interrupts using counter 1 |
| 7 | Program to transfer data serially with baud rate of 9600 |
| 8 | Program to receive data serially with baud rate of 9600 |
| 9 | Sequence Generator Using Serial Interface in 8051 |
| 10 | Programming for Delay Generation and Effect of CPU Clock |
| 11 | Programming with On-Chip Timers/Counters |
| 12 | Programming with On-Chip ADC |
| 13 | Programming for PWM Generation |
| 14 | Programming for I/O Interfacing (LED and Switch Interfacing) |

# Write a program to find the GCD and LCM of a given two bytes in a memory locations

ORG 0000H MOV A, 50H MOV B, 51H BACK: MOV R1,B DIV AB

MOV A, B

CJNE A, #00H, L1 L1: JZ LAST

MOV A, R1 SJMP BACK

LAST: MOV A, R1 MOV 52H, A

MOV A, 50H MOV B, 51H MUL AB MOV B, 52H DIV AB MOV 53H, A END

# Write a program to convert the given BCD number into its equivalent seven segment value.

ORG 0000H MOV P1, #0FFH MOV P2, #00H

MOV DPTR, #0300H AGAIN: MOV A, P1 ANL A, #0FH

MOVC A,@A+DPTR MOV P2, A

SJMP AGAIN ORG 0300H

DB 3FH,06H,5BH,4FH,66H,6DH,7DH,07H,7FH,6FH END

- Toggling of p1.5 with the delay of 5ms without interrupts using timer 1

ORG 0000H

MOV TMOD, #10h AGAIN: MOV TL1, #0FFH MOV TH1, #0EDH

CPL P1.5 SETB TR1

BACK: JNB TF1, BACK CLR TR1

CLR TF1 SJMP AGAIN END

- Toggling of p1.2 with the delay of 5ms with interrupts using timer 0

ORG 0 LJMP MAIN

ORG 000BH; ISR for Timer 0 CPL P1.2

MOV TL0, #0FFH MOV TH0, #0EDH RETI

ORG 30H

MAIN: MOV TMOD, #01h; Timer 0, Mode 1 MOV TL0, #00

MOV TH0, #0DCH

MOV IE, #82H; enable Timer 0 interrupt SETB TR0

HERE: SJMP HERE END

- Count no. 0f pulses without interrupts using counter 0

ORG 0000H

MOV TMOD, #05h SETB P3.4

MOV TL0, #00H MOV TH0, #00H SETB TR0

BACK: MOV A, TL0 MOV P1, A

MOV A, TH0 MOV P2, A SJMP BACK END

- Count no. 0f pulses without interrupts using counter 1

ORG 0000H LJMP MAIN ORG 001BH MOV P1, #55H

MOV TL1, #0FAH MOV TH1, #0FFH

RETI ORG 30H

MAIN: MOV TMOD, #50h SETB P3.5

MOV TL1, #0FAH MOV TH1, #0FFH MOV IE, #88H SETB TR1

HERE: MOV P1, #0AAH SJMP HERE

END

- Program to transfer data serially with baud rate of 9600

MOV TMOD, #20H MOV TH1, #-3 MOV SCON, #50H SETB TR1

AGAIN: MOV SBUF, #”A” HERE: JNB TI, HERE

CLR TI SJMP AGAIN END

- Program to receive data serially with baud rate of 9600

ORG 0000h

MOV TMOD, #20H MOV TH1, #-3 MOV SCON, #50H SETB TR1

HERE: JNB RI, HERE MOV A, SBUF

MOV P1, A CLR RI SJMP HERE END

- Sequence Generator Using Serial Interface in 8051

TX_CHAR EQU 175DH DELAY EQU 6850H ORG 0000H

LJMP MAIN_START ORG 9000H MAIN_START:

MOV R4, #08 MOV A, #01H NXT_CH:

MOV R6, A MOV B, A ADD A, #30H

CJNE A, #3AH, N00 N00:

JC N05

ADD A, #07H N05:

MOV SBUF, A

LCALL UART_TX_CHAR MOV A, B

ADD A, #02H DJNZ R4, NXT_CH SJMP $

ORG 175DH UART_TX_CHAR: JNB TI, $

CLR TI RET END